### (version 20090601)

### Sustainable Public Procurement-fiche: advanced

# 1) Subject matter

Publication paper produced with environmentally friendly materials and processes.

“For <…..> (name of the public authority), the care for the environment and social aspects is important. It is stated in her <strategic policies>, <mission>, <vision>, <procurement policy>, …”

2) Exclusion criteria

Non compliance with environmental and social legislation, which has been the subject of a final judgment or a decision having equivalent effect, may be considered an offence concerning the professional conduct of the economic operator concerned or grave misconduct, permitting to exclude the party concerned from competing for the contract

Ref:

Art. 53 and 54 of Directive 2004/17/EC and Art. 45 of Directive 2004/18/EC

3) Technical capacity(not exclusive)

/

4) Technical specifications

Raw materials

- Waste paper treatment shall be done without the use of chlorine, halogenated bleaching agents and poorly biodegradable complexing agents, such as e.g. ethylenediaminetetraacetic acids (EDTAs) and diethylenetriaminepentaacetic acids (DTPAs).

- Virgin fibres may be used, but the content of virgin fibres doesn't exceed 250 kg per 1000 kg of newly made paper (air dry)

- The origin of the timber used for addition of virgin fibres is verifiable.

- At least 70% of the total virgin fibre content are virgin fibres from sustainable managed forests.

- Virgin fibres may be used only if they are manufactured without the aid of chlorine, halogenated bleaching agents and optical brighteners.

Colorants

- If colorants are used the following requirements are met:

- Azo dyes or pigments which may release one of the amines listed in Directive 2002/61/EEC are not used as colorants.

- Colorants (pigments or dyes) containing mercury, lead, cadmium or hexavalent chromium compounds as constituents are not used.

- No colorants, surface-finishing agents, auxiliaries and coating materials may be used that are classified with any of the following risk phrases, or any combination thereof, in accordance with Directive 1999/45/EC or Directive 67/548/EEC and their amendment:

- R40 (see annex)

- R43

- R45

- R46

- R49

- R60

- R61

- R62

- R63

- R68

Other chemical substances

- The content of detectable formaldehyde in the final product doesn't exceed 0.5mg/dm². The determination shall be made in accordance with EN 1541.

- The content of pentachlorophenol in the final product doesn't exceed 0.15 mg/kg. The determination shall be made in accordance with EN ISO 15320.

- The product is manufactured without the use of glyoxal-containing auxiliaries.

- The addition of optical brighteners is not permitted.

- Only those substances are used as biocides in the manufacture of products which are listed as so-called existing substances in Annex II to Commission Regulation (EC) No. 2032/20038. That means they have to be notified for the respective type of biocidal product and adopted in the EC review program.

- Tetramethylthiuram disulfide (CAS No. 137-26-8) is not used as biocide.

Evidence:

The compliance with all the criteria mentioned above can be proved with the following label:

Blaue Engel

(Only for the criterium ‘raw materials’: or equivalent)

In case that the tendering company can present this label, any further proof is not necessary. Any other suitable evidence from a recognized body can also be used.

The proof of the technical specifications for virgin fibers can be delivered by the following labels: FSC or PEFC (Any other suitable evidence from a recognized body can also be used)

## 5) Awarding the contract:

|  | Criterium | Weight |
| --- | --- | --- |
| 1 | Price Calculation (e.g.): Lowest offered price/ stated price x 0,70 | e.g. 70% |
| 2 | Environmental criteria (The public authority formulates the points it wants to assign to the below mentioned criteria ) Calculation (e.g.): Total scored points / maximum number of points x 0,20 | e.g. 20% |
| 3 | … | e.g. 5 % |
| 4 | … | e.g. …. |

Environmental criteria

/

6) Performance clauses:

/

### References

[Information of the public authority that used these clauses in a procurement case]

Annex R-PHRASES: 
(R-phrases are mentioned on product labels and in product safety datasheets. It can be a useful tool for verification-procedures.)

| R1: | Explosive when dry. |
| --- | --- |
| R2: | Risk of explosion by shock, friction, fire or other sources of ignition. |
| R3: | Extreme risk of explosion by shock, friction, fire or other sources of ignition. |
| R4: | Forms very sensitive explosive metallic compounds. |
| R5: | Heating may cause an explosion. |
| R6: | Explosive with or without contact with air. |
| R7: | May cause fire. |
| R8: | Contact with combustible material may cause fire. |
| R9: | Explosive when mixed with combustible material. |
| R10: | Flammable |
| R11: | Highly flammable |
| R12: | Extremely flammable |
| R13 (obsolet): | Extremely flammable liquid gas (This R-phrase is no longer designated by the version of the GefStoffV published on 26.10.93.) |
| R14: | Reacts violently with water. |
| R15: | Contact with water liberates extremely flammable gases. |
| Merck R15.1 | Contact with acid liberates extremely flammable gases. |
| R16: | Explosive when mixed with oxidizing substances. |
| R17: | Spontaneously flammable in air. |
| R18: | In use, may form flammable/explosive vapour-air mixture. |
| R19: | May form explosive peroxides. |
| R20: | Harmful by inhalation. |
| R21: | Harmful in contact with skin. |
| R22: | Harmful if swallowed. |
| R23: | Toxic by inhalation. |
| Riedel-de Haen R23K: | Also toxic by inhalation. |
| R24: | Toxic in contact with skin. |
| Riedel-de Haen R24K: | Also toxic in contact with skin. |
| R25: | Toxic if swallowed. |
| Riedel-de Haen R25K: | Also toxic if swallowed. |
| R26: | Very toxic by inhalation. |
| Riedel-de Haen R26K: | Also very toxic by inhalation. |
| R27: | Very toxic in contact with skin |
| Riedel-de Haen R27A: | Very toxic in contact with eyes. |
| Riedel-de Haen R27K: | Also very toxic in contact with skin. |
| Riedel-de Haen R27AK: | Also very toxic in contact with eyes. |
| R28: | Very toxic if swallowed. |
| Riedel-de Haen R28K: | Also very toxic if swallowed. |
| R29: | Contact with water liberates toxic gas. |
| R30: | Can become highly flammable in use. |
| R31: | Contact with acids liberates toxic gas. |
| Merck R31.1 | Contact with alkalies liberates toxic gas. |
| R32: | Contact with acids liberates very toxic gas. |
| R33: | Danger of cumulative effects. |
| R34: | Causes burns. |
| R35: | Causes severe burns. |
| R36: | Irritating to eyes. |
| Riedel-de Haen R36A: | Lacrimating |
| R37: | Irritating to respiratory system. |
| R38: | Irritating to skin. |
| R39: | Danger of very serious irreversible effects. |
| R40: | Possible risk of cancer. CAUTION: Until 2001 this R-phrase was used for possible mutagenic or teratogenic risks as well. These risks are now labelled with R68! |
| R41: | Risk of serious damage to eyes. |
| R42: | May cause sensitization by inhalation. |
| R43: | May cause sensitization by skin contact. |
| R44: | Risk of explosion if heated under confinement. |
| R45: | May cause cancer. |
| R46: | May cause heritable genetic damage. |
| R47(obsolet): | May cause deformities. (This R-phrase is no longer designated by the version of the GefStoffV published on 26.10.93.) |
| R48: | Danger of serious damage to health by prolonged exposure. |
| R49: | May cause cancer by inhalation. |
| R50: | Very toxic to aquatic organisms. |
| R51: | Toxic to aquatic organisms. |
| R52: | Harmful to aquatic organisms. |
| R53: | May cause long-term adverse effects in the aquatic environment. |
| R54: | Toxic to flora. |
| R55: | Toxic to fauna. |
| R56: | Toxic to soil organisms. |
| R57: | Toxic to bees. |
| R58: | May cause long-term adverse effects in the environment. |
| R59: | Dangerous for the ozone layer. |
| R60: | May impair fertility. |
| R61: | May cause harm to the unborn child. |
| R62: | Possible risk of impaired fertility. |
| R63: | Possible risk of harm to the unborn child. |
| R64: | May cause harm to breastfed babies. |
| R65: | Harmful: may cause lung damage if swallowed. |
| R66: | Repeated exposure may cause skin dryness or cracking. |
| R67: | Vapours may cause drowsiness and dizziness. |
| R68: | Possible risks of irreversible effects. |

- COMBINATIONS OF R-PHRASES:

| R14/15: | Reacts violently with water, liberating extremely flammable gases. |
| --- | --- |
| R15/29: | Contact with water liberates toxic, extremely flammable gas. |
| R20/21: | Harmful by inhalation and in contact with skin. |
| R21/22: | Harmful in contact with skin and if swallowed. |
| R20/22: | Harmful by inhalation and if swallowed. |
| R20/21/22: | Harmful by inhalation, in contact with skin and if swallowed. |
| R21/22: | Harmful in contact with skin and if swallowed. |
| R23/24: | Toxic by inhalation and in contact with skin. |
| R24/25: | Toxic in contact with skin and if swallowed. |
| R23/25: | Toxic by inhalation and if swallowed. |
| R23/24/25: | Toxic by inhalation, in contact with skin and if swallowed. |
| R24/25: | Toxic in contact with skin and if swallowed. |
| R26/27: | Very toxic by inhalation and in contact with skin. |
| R27/28: | Very toxic in contact with skin and if swallowed. |
| R26/28: | Very toxic by inhalation and if swallowed. |
| R26/27/28: | Very toxic by inhalation, in contact with skin and if swallowed. |
| R36/37: | Irritating to eyes and respiratory system. |
| R37/38: | Irritating to respiratory system and skin. |
| R36/38: | Irritating to eyes and skin. |
| R36/37/38: | Irritating to eyes, respiratory system and skin. |
| R39/23: | Toxic: danger of very serious irreversible effects through inhalation. |
| R39/24: | Toxic: danger of very serious irreversible effects in contact with skin. |
| R39/25: | Toxic: danger of very serious irreversible effects if swallowed. |
| R39/23/24: | Toxic: danger of very serious irreversible effects through inhalation and in contact with skin. |
| R39/23/25: | Toxic: danger of very serious irreversible effects through inhalation and if swallowed. |
| R39/24/25: | Toxic: danger of very serious irreversible effects in contact with skin and if swallowed. |
| R39/23/24/25: | Toxic: danger of very serious irreversible effects through inhalation, in contact with skin and if swallowed. |
| R39/26: | Very toxic: danger of very serious irreversible effects through inhalation. |
| R39/27: | Very toxic: danger of very serious irreversible effects in contact with skin. |
| R39/28: | Very toxic: danger of very serious irreversible effects if swallowed. |
| R39/26/27: | Very toxic: danger of very serious irreversible effects through inhalation and in contact with skin. |
| R39/26/28: | Very toxic: danger of very serious irreversible effects through inhalation and if swallowed. |
| R39/27/28: | Very toxic: danger of very serious irreversible effects in contact with skin and if swallowed. |
| R39/26/27/28: | Very toxic: danger of very serious irreversible effects through inhalation, in contact with skin and if swallowed. |
| R42/43: | May cause sensitization by inhalation and skin contact. |
| R48/20: | Harmful: danger of serious damage to health by prolonged exposure through inhalation. |
| R48/21: | Harmful: danger of serious damage to health by prolonged exposure in contact with skin. |
| R48/22: | Harmful: danger of serious damage to health by prolonged exposure if swallowed. |
| R48/20/21: | Harmful: danger of serious damage to health by prolonged exposure through inhalation and in contact with skin. |
| R48/20/22: | Harmful: danger of serious damage to health by prolonged exposure through inhalation and if swallowed. |
| R48/21/22: | Harmful: danger of serious damage to health by prolonged exposure in contact with skin and if swallowed. |
| R48/20/21/22: | Harmful: danger of serious damage to health by prolonged exposure through inhalation, in contact with skin and if swallowed. |
| R48/23: | Toxic: danger of serious damage to health by prolonged exposure through inhalation. |
| R48/24: | Toxic: danger of serious damage to health by prolonged exposure in contact with skin. |
| R48/25: | Toxic: danger of serious damage to health by prolonged exposure if swallowed. |
| R48/23/24: | Toxic: danger of serious damage to health by prolonged exposure through inhalation and in contact with skin. |
| R48/23/25: | Toxic: danger of serious damage to health by prolonged exposure through inhalation and if swallowed. |
| R48/24/25: | Toxic: danger of serious damage to health by prolonged exposure in contact with skin and if swallowed. |
| R48/23/24/25: | Toxic: danger of serious damage to health by prolonged exposure through inhalation, in contact with skin and if swallowed. |
| R50/53: | Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |
| R51/53: | Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |
| R52/53: | Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |
| R68/20: | Harmful: possible risk of irreversible effects through inhalation. |
| R68/21: | Harmful: possible risk of irreversible effects in contact with skin. |
| R68/22: | Harmful: possible risk of irreversible effects if swallowed. |
| R68/20/21: | Harmful: possible risk of irreversible effects through inhalation and in contact with skin. |
| R68/20/22: | Harmful: possible risk of irreversible effects through inhalation and if swallowed. |
| R68/21/22: | Harmful: possible risk of irreversible effects in contact with skin and if swallowed. |
| R68/20/21/22: | Harmful: possible risk of irreversible effects through inhalation, in contact with skin and if swallowed. |